THE ECO-INDICATOR WORKSHEET

| Production |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| material or process |  |  | amount | (kg) | indicator |  | result |  |
| TOTAL |  |  |  |  |  |  |  |  |
| Use |  |  |  |  |  |  |  |  |
| material or process |  |  | amount |  | indicator |  | result |  |
| TOTAL |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |
| material or process |  |  | amount |  | indicator |  | result |  |
| TOTAL |  |  |  |  |  |  |  |  |

COMPLETED TABLE FOR AN ELECTRIC JUICER

| Production |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| material or process |  |  | amount | (kg) | indicator |  | result |  |
| Polystyrene (PS) |  |  | 0.1 |  | 370 |  | 37 |  |
| High Density Polyethylene (HDPE) |  |  | 0.308 |  | 330 |  | 101.6 |  |
| Low Density Polyethylene (LDPE) |  |  | 0.22 |  | 360 |  | 79.2 |  |
| PolyVinyl Chloride (PVC) |  |  | 0.17425 |  | 240 |  | 41.8 |  |
| Nylon |  |  | 0.004 |  | 240 |  | 0.96 |  |
| Rubber |  |  | 0.002 |  | 360 |  | 0.72 |  |
| Steel |  |  | 0.01025 |  | 86 |  | 0.8815 |  |
| Copper |  |  | 0.032 |  | 1400 |  | 44.8 |  |
| Cardboard |  |  | 0.1505 |  | 69 |  | 10.3845 |  |
| Paper |  |  | 0.01 |  | 96 |  | 0.96 |  |
| Injection moulding - 1 |  |  | 0.41 |  | 21 |  | 0.861 |  |
| Injection moulding - 2 |  | (PVC) | 0.17425 |  | 44 |  | 7.667 |  |
| TOTAL |  |  |  |  |  |  | 406.06 |  |
| Use |  |  |  |  |  |  |  |  |
|  |  |  | amount |  | indicator |  | result |  |
| electricity | (kwh) |  | 1.217 |  | 33 |  | 40.2 |  |
| Shipping of product (tkm) |  |  | 12.1tkm |  | 1.1 |  | 13.31 |  |
| 1.1/1000 x 11000 |  |  |  |  |  |  |  |  |
| Distribution (tkm) |  |  | 0.22tkm |  | 15 |  | 3.3 |  |
| 1.1/1000 x 200 |  |  |  |  |  |  |  |  |
| Transport of oranges |  | (tkm) | 1560 |  | 15 |  | 23406 |  |
| (0.57 x 365 x 5)/1000 x 1500 |  |  |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  | 23462.8 |  |
| Disposal |  |  |  |  |  |  |  |  |
| material or process |  |  | amount |  | indicator |  | result |  |
| Landfill |  |  |  |  |  |  |  |  |
| Polystyrene (PS) |  |  | 0.1 |  | 4.1 |  | 0.41 |  |
| High Density Polyethylene (HDPE) |  |  | 0.308 |  | 3 |  | 0.924 |  |
| Low Density Polyethylene (LDPE) |  |  | 0.22 |  | 3 |  | 0.66 |  |
| PolyVinyl Chloride (PVC) |  |  | 0.7425 |  | 2.8 |  | 2.1 |  |
| Nylon |  |  | 0.004 |  | 3.6 |  | 0.0144 |  |
| Steel |  |  | 0.01025 |  | 1.4 |  | 0.0143 |  |
| Copper |  |  | 0.032 |  | 1.4 |  | 0.0448 |  |
| Recycling |  |  |  |  |  |  |  |  |
| Cardboard |  |  | 0.1505 |  | -8.3 |  | -8.15 |  |
| Paper |  |  | 0.01 |  | -1.2 |  | -1.19 |  |
| TOTAL |  |  |  |  |  |  | -5.1725 |  |